# OSHA Ladder & Walking-Working Surfaces Safety Program

Covers: 29 CFR 1910.22, 1910.23, 1910.30

## 1. Program Purpose

The purpose of this safety program is to ensure all employees are protected from hazards related to walking-working surfaces and ladder use, in accordance with OSHA regulations. The program outlines inspection, maintenance, and training protocols to prevent slips, trips, falls, and other related injuries.

## 2. Scope

This program applies to all employees, contractors, and visitors who may work on or around:

- Fixed and portable ladders

- Elevated work surfaces

- Stairways and ramps

- Platforms, mezzanines, scaffolding

- Any walking-working surface

## 3. Regulatory References

- 29 CFR 1910.22 – General requirements for walking-working surfaces

- 29 CFR 1910.23 – Ladders

- 29 CFR 1910.30 – Training requirements

## 4. Roles and Responsibilities

### A. Employer

- Ensure surfaces are maintained in a safe condition.

- Provide appropriate ladders and equipment.

- Implement inspection and maintenance procedures.

- Provide and document training under 1910.30.

### B. Supervisors

- Enforce safe ladder and walking surface use.

- Conduct regular safety audits and inspections.

- Ensure employee training is up to date.

### C. Employees

- Follow safety procedures and report unsafe conditions.

- Use only ladders and equipment that are in good condition.

- Attend all required training.

## 5. General Walking-Working Surface Safety (1910.22)

- Surfaces must be kept clean, orderly, and in a sanitary condition.

- No hazards such as loose boards, wet spots, or clutter.

- Provide safe access and egress to work areas.

- Surfaces must be designed, constructed, and maintained to support intended loads.

- Work areas must be inspected regularly, and unsafe conditions corrected immediately.

## 6. Ladder Safety Requirements (1910.23)

### A. General Ladder Requirements

- Ladders must be capable of supporting intended loads.

- Defective ladders must be tagged “Do Not Use” and removed from service.

- Ladders must have secure footing, and users must maintain 3 points of contact.

- Top step or cap of a stepladder must not be used as a step.

### B. Portable Ladders

- Must have non-slip bases.

- Must be set at a 4:1 angle (1 foot out for every 4 feet of height).

- Must extend at least 3 feet above landing surfaces.

### C. Fixed Ladders

- Must have rest platforms every 150 feet of height.

- Fall protection required for climbs over 24 feet.

- Fixed ladders installed after Nov. 19, 2018 must have a personal fall arrest system (PFAS) or ladder safety system.

## 7. Training Requirements (1910.30)

### A. Who Must Be Trained

- Any employee who uses:

  - Ladders

  - Scaffolds

  - Walking-working surfaces with fall hazards

### B. Training Topics

- Nature of fall hazards

- Correct procedures for using ladders and surfaces

- How to inspect equipment

- Proper use of fall protection systems

- Emergency response in case of a fall

### C. Training Frequency

- Initial training before use

- Retraining required when:

  - Changes in equipment or procedures occur

  - The employee demonstrates inadequate knowledge or unsafe behavior

### D. Documentation

Training records must include:

- Employee name

- Trainer name

- Date of training

- Topics covered

## 8. Inspections and Maintenance

- Daily pre-use ladder inspections required by the user

- Monthly documented inspections by a competent person

- Records of inspections and maintenance must be kept for at least 1 year

## 9. Enforcement

Failure to comply with this safety program may result in:

- Verbal or written warnings

- Retaining or removal from job tasks

- Disciplinary action, up to and including termination

## 10. Program Review

This program shall be reviewed annually or:

- After an incident involving walking-working surfaces

- Upon updates to OSHA standards

- When new equipment or job tasks are introduced

## 11. Appendices (Optional)

- Ladder Inspection Checklist

- Employee Training Sign-in Sheet

## Appendix A: Ladder Inspection Checklist

(Based on 29 CFR 1910.23)
Use for daily or monthly inspections by a competent person.

| Inspection Item | Pass / Fail | Comments / Actions Required |
| --- | --- | --- |
| Ladder is clean and free of oil, grease, or debris | ☐ Pass ☐ Fail |  |
| Steps/rungs are free from damage or excessive wear | ☐ Pass ☐ Fail |  |
| Side rails are not bent, cracked, or split | ☐ Pass ☐ Fail |  |
| All hardware (bolts, rivets, screws) is present and tight | ☐ Pass ☐ Fail |  |
| Ladder feet are secure, clean, and non-slip pads intact | ☐ Pass ☐ Fail |  |
| Ladder is not wobbly or unstable | ☐ Pass ☐ Fail |  |
| Labels and warnings are present and legible | ☐ Pass ☐ Fail |  |
| Spreaders/locking mechanisms function properly | ☐ Pass ☐ Fail |  |
| Ladder is stored correctly (dry, secured, not overloaded) | ☐ Pass ☐ Fail |  |
| No unauthorized repairs or modifications | ☐ Pass ☐ Fail |  |

Inspection Type:
☐ Daily Pre-Use  ☐ Monthly  ☐ After Incident

Ladder ID / Location: ___________________________

Inspected By: ______________________  Date: ______________

Action Taken (if any): ___________________________________________________

## Appendix B: Ladder & Walking Working Surfaces Safety Training Sign-In Sheet

(Covers 1910.22, 1910.23, 1910.30)

Training Topic: Ladder & Walking-Working Surface Safety
Trainer Name: _______________________  Date: ______________
Location: ___________________________

### 📝 Training Topics Covered:

- OSHA ladder regulations (1910.23)

- Walking-working surface safety (1910.22)

- Fall hazard recognition and prevention

- Ladder inspection and proper use

- Personal fall protection systems

- Emergency procedures and reporting

- Hands-on ladder setup and demonstration

### 📌 Attendee Sign-In

| # | Employee Name | Job Title | Signature | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |